Project Documentation

Stepper Motor Precision Positioner

Student Details

Name: Sangili Murugan M

Course: BE Electronics and Communication Engineering

College: Arjun College of Technology

Program: IIT Jammu Summer School 2026 – IoT & Drones

1. Project Title

Stepper Motor Precision Positioner

2. Objective

The objective of this project is to design and implement a precision positioning system using an Arduino Uno and a 28BYJ-48 stepper motor. The system allows users to control the motor position accurately using push buttons while monitoring the current position on an OLED display and Serial Monitor.

3. Problem Statement

Many automation systems such as CNC machines, robotic arms, and conveyor systems require precise motor positioning. This project demonstrates how a stepper motor can be controlled accurately using Arduino, making it suitable for industrial automation and embedded system applications.

4. Components Required

- Arduino Uno R3

- 28BYJ-48 Stepper Motor

- ULN2003 Stepper Driver (simulated in Wokwi)

- OLED Display (SSD1306 128×64)

- 10kΩ Potentiometer

- Three Push Buttons

- LED

- 220Ω Resistor

- Breadboard

- Jumper Wires

5. Circuit Connections

| Component | Arduino Pin |
| --- | --- |
| Potentiometer | A0 |
| CW Button | D2 |
| CCW Button | D3 |
| HOME Button | D4 |
| Status LED | D7 |
| Stepper IN1 | D8 |
| Stepper IN2 | D9 |
| Stepper IN3 | D10 |
| Stepper IN4 | D11 |
| OLED SDA | A4 |
| OLED SCL | A5 |

6. Working Principle

The Arduino continuously reads the potentiometer value and converts it into a target angle between 0° and 360°.

The stepper motor is controlled using three push buttons:

- CW Button rotates the motor clockwise by 256 steps.

- CCW Button rotates the motor counter-clockwise by 256 steps.

- HOME Button resets the current position to zero.

The OLED display continuously shows:

- Current Angle

- Target Angle

- Total Steps

- Motor Direction

The same information is also displayed on the Serial Monitor for debugging and monitoring.

7. Algorithm

- Initialize Arduino, OLED display, and Stepper Motor.

- Read potentiometer value.

- Convert potentiometer value into target angle.

- Check CW button.

  - Rotate motor clockwise.

- Check CCW button.

  - Rotate motor counter-clockwise.

- Check HOME button.

  - Reset current position.

- Update OLED display.

- Print motor information to Serial Monitor.

- Repeat continuously.

8. Program Description

The Arduino Stepper Library is used to control the 28BYJ-48 stepper motor.

The OLED display is driven using the Adafruit SSD1306 library.

The potentiometer provides the desired target angle, while the buttons provide manual motor movement.

The program maintains a variable called current Steps to calculate the current motor angle.

The direction of rotation is stored as a string and displayed on both the OLED display and the Serial Monitor.

9. Wokwi Simulation

Simulation Link

https://wokwi.com/projects/470222613636943873

10. Expected Output

OLED Display

- Current Angle

- Target Angle

- Total Steps

- Direction (CW / CCW / HOME)

Serial Monitor

Example:

Current Angle : 45

Target : 180

Steps : 256

Direction : CW

Current Angle : 90

Target : 270

Steps : 512

Direction : CW

Current Angle : 45

Target : 120

Steps : 256

Direction : CCW

11. Applications

- Robotic Arms

- CNC Machines

- 3D Printers

- Camera Positioning Systems

- Conveyor Belt Automation

- Industrial Position Control

- Educational Embedded Systems

12. Advantages

- High positioning accuracy

- Simple hardware implementation

- Low cost

- Easy to program using Arduino

- Suitable for automation applications

- Real-time monitoring using OLED display

13. Challenges Faced

- Correct sequencing of stepper motor pins.

- Calculating motor angle from total steps.

- Synchronizing OLED display with motor movement.

- Handling button debounce to avoid multiple unwanted rotations.

14. Future Enhancements

- Automatic angle positioning using the potentiometer.

- Speed control using PWM.

- Wireless control using Bluetooth or Wi-Fi.

- Mobile application integration.

- IoT monitoring using MQTT.

- Position memory using EEPROM.

15. Conclusion

The Stepper Motor Precision Positioner project was successfully implemented using Arduino Uno, OLED display, push buttons, and a 28BYJ-48 stepper motor. The system accurately controls motor movement and continuously displays motor position and direction. This project demonstrates the practical application of embedded systems in precision motor control and provides a strong foundation for advanced automation projects.